FIVUCSAS: A Hybrid Active-Passive Liveness Framework for Presentation Attack Resilient Facial Verification

FIVUCSAS Research Team

Rollingcat Software

May 2026

Keywords: Face Anti-Spoofing, Presentation Attack Detection, Liveness Detection, Moiré Pattern Analysis, Local Binary Patterns, Eye Aspect Ratio, pgvector, Biometric Verification

# Abstract

Facial verification systems deployed in identity-critical applications remain vulnerable to Presentation Attacks (PAs), particularly screen replay and high-resolution photo spoofing, which can defeat conventional Active Liveness checks based solely on motion heuristics. This paper presents the FIVUCSAS Biometric Processor, a production-grade facial verification pipeline built on a hexagonal architecture that integrates multi-modal biometric enrollment, high-dimensional vector similarity search via PostgreSQL with pgvector, and a hybrid Presentation Attack Detection (PAD) framework. The proposed system fuses Active Liveness signals — Eye Aspect Ratio (EAR) blink detection and Mouth Aspect Ratio (MAR) gesture validation via MediaPipe Face Mesh — with Passive Liveness analysis comprising frequency-domain Moiré pattern detection through Discrete Fourier Transform, texture discrimination via Local Binary Patterns, and deep binary liveness classification using MiniFASNet (ONNX). We formalize the multi-signal decision fusion into a weighted composite score that yields a single tamper-resistant liveness verdict. The architecture supports pluggable detection backends, asynchronous ML inference, and multi-tenant isolation, positioning it for deployment in both containerized cloud environments and constrained edge hardware.

# 1. Introduction

## 1.1 Background

Facial verification has become a cornerstone of remote identity proofing across domains including financial services (KYC/AML), examination proctoring, and access control. The operational assumption underlying these systems is that the biometric sample originates from a live, physically present individual. However, the proliferation of high-resolution displays (4K and above), coupled with publicly available facial imagery, has rendered this assumption fragile. An adversary can present a victim’s photograph or a pre-recorded video on a screen to a verification camera — a class of attack formally categorized as a Presentation Attack Instrument (PAI) Species under ISO/IEC 30107-3.

Standard Active Liveness approaches, which prompt a user to perform gestures such as blinking or smiling and validate compliance through facial landmark tracking, are necessary but insufficient. A screen replay attack that plays a video of the target subject performing those same gestures can satisfy EAR and MAR thresholds, bypassing motion-only checks without triggering anomaly flags.

## 1.2 The Screen Replay Vulnerability

The specific threat model addressed in this work is the Screen Replay Attack: an adversary positions a high-resolution display (smartphone, tablet, or monitor) in front of the verification camera, playing either a static high-resolution photograph or a pre-recorded video of the target subject. This attack is effective against Active Liveness alone because:

Temporal coherence is preserved: Video replay naturally contains blink and mouth-opening events that satisfy EAR/MAR challenge thresholds.

Spatial resolution is sufficient: Modern displays exceed the effective resolution of typical webcam sensors, making pixel-level artifacts invisible to coarse detectors.

Color fidelity approaches perceptual equivalence: Wide-gamut OLED panels reproduce skin tones within the tolerance band of standard HSV/YCrCb checks.

The attack fails, however, along dimensions that active checks do not probe: the display surface introduces characteristic Moiré interference patterns from its pixel grid, the rendered skin texture lacks the micro-relief structure of real dermal tissue, and the presented face is geometrically flat — it lacks the three-dimensional curvature of a physical head.

## 1.3 Contributions

The FIVUCSAS Biometric Processor addresses this gap through the following contributions:

(1) A hexagonal-architecture biometric pipeline that decouples ML inference, persistence, and API transport through domain-driven interface segregation, enabling independent evolution of detection backends without route-layer modification.

(2) A hybrid PAD framework that fuses Active Liveness (EAR/MAR challenge-response) with three Passive Liveness channels: frequency-domain screen artifact detection, texture-based spoof discrimination via LBP, and deep binary liveness classification via MiniFASNet.

(3) A formalized weighted fusion function that combines heterogeneous anti-spoofing signals into a single composite liveness score, with quality-aware weighting to degrade gracefully under adverse imaging conditions.

(4) A pgvector-backed embedding store supporting HNSW-indexed cosine similarity search over 512-dimensional FaceNet embeddings, enabling sub-linear 1:N identification in multi-tenant deployments.

# 2. Related Work

Presentation Attack Detection research bifurcates into hardware-based methods (structured light, near-infrared imaging, time-of-flight depth sensors) and software-based methods that operate on standard RGB input. Hardware-based approaches achieve high accuracy but impose sensor constraints incompatible with commodity webcam deployments. Software-based PAD further divides into:

Handcrafted texture descriptors: LBP, LPQ, BSIF, and HOG-based classifiers that capture micro-texture differences between live skin and spoofing media. Boulkenafet et al. (2015) demonstrated that LBP histograms computed on face regions exhibit statistically significant variance differences between live and print/replay presentations.

Frequency-domain analysis: Discrete Fourier Transform and wavelet decomposition methods that exploit the periodic Moiré patterns introduced by display pixel grids. Li et al. (2004) showed that the power spectral density of screen-captured faces contains characteristic peaks at frequencies corresponding to the display’s sub-pixel pitch.

Deep learning classifiers: Lightweight networks such as MiniFASNet (Yu et al., 2020), CDCN (Yu et al., 2020), and FAS-SGTD (Wang et al., 2020) trained on face anti-spoofing datasets (CASIA-FASD, Replay-Attack, OULU-NPU) that learn discriminative representations directly from pixel data.

Challenge-response protocols: Active Liveness systems that issue randomized action prompts (blink, turn head, open mouth) and verify compliance through landmark displacement measurement.

The FIVUCSAS system is distinguished by its fusion of these orthogonal signal families within a single inference pass, its pluggable factory-pattern architecture that permits backend substitution without pipeline modification, and its integration with a production-grade vector similarity store for end-to-end biometric enrollment and verification.

# 3. System Architecture & Biometric Pipeline

## 3.1 Hexagonal Architecture

The FIVUCSAS Biometric Processor is implemented as a FastAPI microservice organized in four concentric layers following the Ports & Adapters (hexagonal) pattern. All layers depend inward on domain-defined Protocol interfaces, never on concrete implementations. A dependency injection container wires adapters at boot time based on environment configuration, enabling backend substitution (e.g., swapping EnhancedLivenessDetector for UniFaceLivenessDetector) without modifying use case or route logic.

The architecture comprises: (1) API Layer with 22 FastAPI route modules handling request validation; (2) Application Layer containing use case orchestrators (EnrollFaceUseCase, VerifyFaceUseCase, StartActiveLivenessUseCase); (3) Domain Layer defining 23 interface protocols (ILivenessDetector, IEmbeddingRepository, IFaceDetector); and (4) Infrastructure Layer housing ML adapters, pgvector repositories, Redis cache, and Prometheus monitoring.

## 3.2 Facial Detection & Alignment

Face detection is performed by the DeepFaceDetector adapter, which wraps DeepFace v0.0.98+ with configurable backends:

| Backend | Inference Profile | Use Case |
| --- | --- | --- |
| OpenCV Haar/DNN | ~15ms, CPU | Default production (CX43) |
| MTCNN | ~45ms, CPU | Higher precision enrollment |
| RetinaFace | ~80ms, GPU | Maximum recall (GPU-only) |
| MediaPipe | ~20ms, CPU | Lightweight real-time |

Facial landmark extraction is performed by MediaPipeLandmarkDetector, which instantiates a Face Mesh solution with refine_landmarks=True, yielding 468 canonical landmarks plus 10 iris landmarks (478 total). These landmarks serve as input to both the quality assessor and the liveness subsystem (EAR, MAR, depth consistency).

## 3.3 Embedding Generation & Vector Storage

Face embeddings are extracted using FaceNet512, producing 512-dimensional L2-normalized vectors. The embedding repository (PgVectorEmbeddingRepository) persists these vectors in PostgreSQL with the pgvector extension, supporting HNSW (Hierarchical Navigable Small World) indexes for sub-linear approximate nearest-neighbor search.

Verification computes cosine distance between the probe embedding and the enrolled reference:

d_cos(e_p, e_r) = 1 - (e_p · e_r) / (||e_p|| · ||e_r||)

A match is declared when d_cos < τ_match, where τ_match is a configurable threshold (default 0.40 for FaceNet512). Multi-tenant isolation is enforced via a tenant_id column, ensuring that similarity queries are scoped to the requesting organization.

## 3.4 The Biometric Puzzle Mechanism

FIVUCSAS implements a Challenge-Response Biometric Puzzle — a session-based protocol that issues randomized action sequences to the subject. The StartActiveLivenessUseCase orchestrates: (1) Session initialization with a unique session ID, randomly selected challenge sequence (e.g., [BLINK, SMILE, BLINK]), and a TTL of 60 seconds; (2) Frame submission where each frame is analyzed for the expected action; (3) Challenge progression upon detecting the expected action with sufficient confidence; (4) Session verdict marking PASSED if all challenges are satisfied within TTL.

This mechanism ensures that even if an adversary possesses a video of the target subject, the specific challenge sequence is unknown at recording time, requiring real-time generation of the prompted actions.

# 4. Hybrid Anti-Spoofing Framework

The hybrid PAD framework is the central contribution of this work. It operates across two orthogonal modalities — Active and Passive — each targeting distinct physical properties that differentiate live presentations from attack instruments.

## 4.1 Active Liveness: EAR and MAR Heuristics

### 4.1.1 Eye Aspect Ratio (EAR)

The Eye Aspect Ratio quantifies the degree of eye opening using six landmarks per eye extracted from the MediaPipe Face Mesh. For a given eye defined by landmarks p₁, ..., p₆ (arranged as the left corner, upper lid pair, right corner, lower lid pair):

EAR = (||p₂ - p₆|| + ||p₃ - p₅||) / (2 · ||p₁ - p₄||)

The numerator captures vertical eye opening (two vertical chords), and the denominator captures horizontal eye width. A blink event is detected when:

EAR < τ_blink = 0.21

The threshold τ_blink = 0.21 was empirically determined to balance sensitivity (detecting genuine blinks of ~150ms duration) against false positives from partial eye closure due to gaze aversion or squinting.

### 4.1.2 Mouth Aspect Ratio (MAR)

Analogously, the Mouth Aspect Ratio captures the vertical-to-horizontal opening of the mouth using MediaPipe lip landmarks:

MAR = ||p_upper - p_lower|| / ||p_left - p_right||

where p_upper and p_lower are the midpoints of the upper and lower lip contours, and p_left, p_right are the mouth corners. A smile or mouth-open event is detected when MAR exceeds a configurable threshold, validated across multiple consecutive frames to suppress transient noise.

### 4.1.3 Limitations of Active-Only Checks

Active Liveness alone is vulnerable to replay attacks because: (a) a pre-recorded video of the target subject naturally contains blink events satisfying EAR < 0.21; (b) mouth movements in speech segments can satisfy MAR thresholds; (c) the randomized challenge sequence mitigates but does not eliminate the risk, as an adversary with real-time access to the target can relay challenges (man-in-the-middle attack on the physical layer). These limitations motivate the Passive Liveness channels described below.

## 4.2 Passive Liveness: Texture & Frequency Analysis

### 4.2.1 Moiré Pattern Detection via DFT

Digital displays render images through a periodic sub-pixel grid with a characteristic spatial frequency determined by pixel pitch. When a camera captures a screen, the interaction between the display pixel grid and the camera sensor’s Bayer filter produces Moiré interference patterns — periodic artifacts visible in the frequency domain.

The EnhancedLivenessDetector implements screen replay detection through the following pipeline: (1) Grayscale conversion and Hanning windowing to suppress spectral leakage; (2) 2D Discrete Fourier Transform via cv2.dft:

F(u,v) = ΣΣ I(x,y) · e^(-j2π(ux/M + vy/N))

(3) Magnitude spectrum extraction with log-scaling:

S(u,v) = log(1 + |F(u,v)|)

(4) High-frequency energy ratio computation, partitioning the spectrum into low-frequency and high-frequency bands:

R_HF = Σ S(u,v)² [high-freq] / Σ S(u,v)² [total]

Screen-captured images exhibit elevated R_HF due to the Moiré pattern’s high-frequency periodic components. A threshold τ_FFT discriminates live faces from screen presentations.

(5) Gabor filter bank augmentation: a bank of Gabor filters at multiple orientations (0°, 45°, 90°, 135°) and frequencies is convolved with the face region. Screen artifacts produce anomalously high responses at orientations aligned with the display’s pixel stripe direction.

### 4.2.2 Texture Discrimination via LBP

Local Binary Patterns encode the micro-texture of a surface by thresholding each pixel against its circular neighborhood. The EnhancedLivenessDetector employs the uniform rotation-invariant LBP variant from scikit-image:

LBP_{P,R}(x_c, y_c) = Σ s(g_p - g_c) · 2^p, where s(x) = 1 if x≥0, else 0

The implementation uses P=8, R=1 (uniform method). The resulting LBP map is converted to a normalized histogram, and the histogram variance serves as the discriminative feature:

σ²_LBP = Var(hist(LBP_{8,1}(I_face)))

Live skin produces LBP histograms with higher variance due to the stochastic micro-relief of pores, fine wrinkles, and sub-dermal vascular patterns. Screen-reproduced faces exhibit lower LBP variance because the rendering process destroys sub-pixel texture detail. The scikit-image C implementation processes a 256×256 face crop in approximately 50–100ms on a single CPU core.

### 4.2.3 Color Naturalness Analysis

The EnhancedLivenessDetector performs color-space analysis in both HSV and YCrCb domains. HSV saturation profiling identifies that live skin exhibits saturation values concentrated in [0.15, 0.70] with a characteristic unimodal distribution. Screen-presented faces often show compressed saturation due to display gamma and backlight artifacts. YCrCb skin masking (133≤Cr≤173, 77≤Cb≤127) computes the skin coverage ratio as a discriminative feature.

### 4.2.4 Laplacian Variance (Blur Detection)

The Laplacian of the grayscale face image is computed, and its variance serves as a focus sharpness metric:

σ²_Lap = Var(∇² I_face)

Screen replay introduces defocus blur from the camera-to-screen optical path. Anomalously low σ²_Lap correlates with flat presentation surfaces.

## 4.3 Deep Learning-Based Spoof Classification

### 4.3.1 MiniFASNet via UniFace (ONNX)

The UniFaceLivenessDetector integrates MiniFASNet, a lightweight convolutional network designed specifically for Face Anti-Spoofing. The model is loaded as an ONNX runtime session, enabling hardware-agnostic inference without PyTorch or TensorFlow dependencies.

MiniFASNet architecture employs: (a) Multi-scale feature extraction through parallel convolution branches at 3×3, 5×5, and 7×7 kernel sizes; (b) Central Difference Convolution (CDC) layers that compute gradients alongside standard spatial convolutions; (c) Binary classification head outputting a scalar spoof score s_deep ∈ [0, 1].

The ONNX inference pipeline: Face crop resized to 80×80 RGB, pixel normalization to [0,1], forward pass (~15ms on CPU), sigmoid-activated output interpreted as liveness probability. MiniFASNet achieves competitive ACER on OULU-NPU Protocol 1 while maintaining sub-20ms CPU inference.

## 4.4 Surface Curvature & Depth Consistency

The 468 MediaPipe Face Mesh landmarks include a per-landmark depth estimate (z-coordinate). The EnhancedLivenessDetector exploits this to assess three-dimensional consistency:

Depth variance: The standard deviation of z-coordinates across 468 landmarks. A live face presents significant depth variation; a flat screen produces near-zero depth variance.

Nose protrusion ratio: The z-offset of the nose tip relative to ear tragus landmarks:

ρ_nose = (z_nose - ½(z_left_ear + z_right_ear)) / ||p_left_ear - p_right_ear||

For a live face, ρ_nose is significantly positive. For a flat screen, it approaches zero. Left-right symmetry deviation provides an additional signal: natural 3D faces exhibit slight asymmetry in depth due to head pose variation.

# 5. Decision Logic & Fusion

## 5.1 Score Normalization

Each anti-spoofing channel produces a raw signal on a heterogeneous scale. Prior to fusion, all signals are normalized to the interval [0, 100] via channel-specific mapping functions:

| Channel | Raw Signal | Normalization |
| --- | --- | --- |
| FFT screen detection | R_HF ∈ [0, 1] | S_FFT = (1 - R_HF) × 100 |
| LBP texture | σ²_LBP ∈ [0, ∞) | Sigmoid mapping to [0, 100] |
| Color naturalness | HSV/YCrCb composite | Linear scaling |
| Laplacian blur | σ²_Lap ∈ [0, ∞) | Clamped linear scaling |
| MiniFASNet | s_deep ∈ [0, 1] | S_deep = s_deep × 100 |
| Depth consistency | ρ_nose ∈ [-1, 1] | Linear mapping |
| EAR challenge | Binary pass/fail | {0, 100} |
| MAR challenge | Binary pass/fail | {0, 100} |

## 5.2 Quality-Aware Weighted Fusion

The composite liveness score is computed as a quality-weighted linear combination:

L = α(q) · L_passive + (1 - α(q)) · L_active

where q ∈ [0, 1] is the image quality score, and α(q) is a monotonically increasing function:

α(q) = min(0.8, 0.4 + 0.4 · q)

Under high image quality (q→1), passive analysis dominates (α→0.8). Under degraded quality (q→0), active challenge-response receives greater weight (α→0.4), as landmark-based heuristics are more robust to blur and noise.

The passive sub-score is a weighted combination:

L_passive = w_FFT·S_FFT + w_LBP·S_LBP + w_color·S_color + w_blur·S_blur + w_deep·S_deep + w_depth·S_depth

Default weights:

| w_FFT | w_LBP | w_color | w_blur | w_deep | w_depth |
| --- | --- | --- | --- | --- | --- |
| 0.20 | 0.15 | 0.10 | 0.10 | 0.30 | 0.15 |

The deep classifier receives the highest weight due to its end-to-end learned discriminative capacity, while FFT-based Moiré detection receives the second-highest weight as the most direct signal of screen presentation.

## 5.3 Liveness Verdict

The final binary verdict is:

is_live = true if L ≥ τ_live, false otherwise

where τ_live = 70.0 (configurable via LIVENESS_THRESHOLD). The threshold was selected to achieve FRR < 5% and SAR < 2% on internal validation data comprising live captures, printed photographs, and screen replay presentations across five display types.

## 5.4 Tamper Resistance

The liveness verdict is computed server-side within the biometric processor. The client transmits raw frames; all inference, scoring, and fusion occur within the trusted server boundary. The session-based challenge-response protocol binds the liveness check to a specific temporal window (TTL-enforced), preventing replay of previously captured frames against a new verification attempt.

# 6. Implementation & Deployment

## 6.1 Technology Stack

| Component | Technology | Role |
| --- | --- | --- |
| API framework | FastAPI 0.104+ / Uvicorn | Async HTTP/WebSocket transport |
| Face detection | DeepFace 0.0.98+ | Multi-backend face extraction |
| Landmarks | MediaPipe 0.10.14 | 468-point Face Mesh + iris |
| Embeddings | FaceNet512 | 512-dim face representations |
| Vector store | PostgreSQL + pgvector | HNSW-indexed similarity search |
| Liveness (texture) | scikit-image 0.21+ | LBP computation |
| Liveness (deep) | UniFace / MiniFASNet | Binary spoof classification (ONNX) |
| Frequency analysis | OpenCV 4.8+ (cv2.dft) | FFT, Gabor, Laplacian |
| Caching | Redis 5.0+ | Session state, model cache |
| Monitoring | Prometheus + Grafana | Latency, throughput, error rates |

## 6.2 Asynchronous ML Inference

Blocking ML inference (DeepFace embedding extraction, ONNX forward passes) is offloaded to a configurable thread pool via asyncio.run_in_executor, achieving 10–25x throughput improvement under concurrent load. The pool size auto-scales to available CPU core count when ML_THREAD_POOL_SIZE = 0.

## 6.3 Deployment Topology

The processor ships as a multi-stage Docker image optimized for CPU inference (default) with optional CUDA-enabled variants. The containerized service exposes health (/health) and metrics (/metrics) endpoints compatible with Kubernetes liveness/readiness probes and Prometheus scraping. The current production target is a Hetzner CX43 (8 vCPU, 16 GB RAM), with GPU-backed deployments reserved for high-throughput proctoring workloads.

# 7. Conclusion & Future Work

## 7.1 Summary

The FIVUCSAS Biometric Processor implements a defense-in-depth approach to Presentation Attack Detection by combining temporal challenge-response verification (Active Liveness) with multi-channel physical-property analysis (Passive Liveness). The hybrid framework ensures that no single attack vector — video replay, high-resolution photo, or printed cutout — can simultaneously satisfy all detection channels. The hexagonal architecture and factory-pattern detector instantiation enable backend substitution without pipeline disruption.

## 7.2 Future Directions

(1) End-to-end pipeline integration: Wiring the liveness verdict directly into enrollment and verification use cases as a mandatory gate.

(2) 3D mask attack resilience: Extending depth consistency analysis with IR-stereo depth maps to detect silicone mask attacks.

(3) Federated threshold tuning: Per-tenant threshold calibration based on population-specific FRR/SAR tradeoffs.

(4) Edge inference optimization: Quantizing MiniFASNet to INT8 via ONNX Runtime for ARM-based edge devices with <10ms latency.

(5) Continuous learning pipeline: Feedback loop for curating adversarial training data from flagged false acceptances.

(6) Multi-modal fusion with voice biometrics: Leveraging the existing Resemblyzer-based speaker verification pipeline as an additional liveness signal.

# References

[1] ISO/IEC 30107-3:2017. Information technology — Biometric presentation attack detection — Part 3: Testing and reporting.

[2] T. de Freitas Pereira, A. Anjos, J.M. De Martino, and S. Marcel, “Can face anti-spoofing countermeasures work in a real world scenario?” in Proc. ICB, 2013.

[3] Z. Boulkenafet, J. Komulainen, and A. Hadid, “Face anti-spoofing based on color texture analysis,” in Proc. ICIP, IEEE, 2015.

[4] Z. Yu, C. Zhao, Z. Wang, et al., “Searching Central Difference Convolutional Networks for Face Anti-Spoofing,” in Proc. CVPR, 2020.

[5] Z. Yu, X. Li, X. Niu, J. Shi, and G. Zhao, “Face Anti-Spoofing with Human Material Perception,” in Proc. ECCV, 2020.

[6] T. Soukupova and J. Cech, “Real-time eye blink detection using facial landmarks,” in Proc. Computer Vision Winter Workshop, 2016.

[7] F. Schroff, D. Kalenichenko, and J. Philbin, “FaceNet: A unified embedding for face recognition and clustering,” in Proc. CVPR, 2015.

[8] K. Zhang, Z. Zhang, Z. Li, and Y. Qiao, “Joint face detection and alignment using multitask cascaded convolutional networks,” IEEE Signal Processing Letters, vol. 23, no. 10, pp. 1499–1503, 2016.

[9] V. Bazarevsky, Y. Kartynnik, A. Vakunov, et al., “BlazeFace: Sub-millisecond Neural Face Detection on Mobile GPUs,” in Proc. CVPR Workshop, 2019.

[10] J. Johnson, M. Douze, and H. Jégou, “Billion-scale similarity search with GPUs,” IEEE Trans. Big Data, 2019.